Creating a block model (Machine vice Base)

Creating a block

- Create new blank workspace using standard mm ipt.

- Start new 2d sketch.

- Select X-Y plane.

- Select 2 point rectangular tool and create rectangle from centre point (type in dimensions) X first(enter dimension value) > TAB >Y second( enter dimension value) > enter.

- Finish sketch.

- Select extrude tool.

- Extrude rectangle 10mm in forward direction.

Creating holes in a block

- Start new 2d sketch on front face of rectangle.

- Select point tool.

- Select 2 points on the block, right click, press ok.

- Select dimension tool.

- Position points using dimension tool to correct positions as marked on drawing provided for 2 M6 threaded holes.

- Finish sketch.

- Select hole tool.

- Change features on hole tool to:

- Type – Tapped hole.

- Seat – None.

- Threads:

  - Type – ISO metric profile.

  - Size – 6mm.

  - Pitch – M6x1mm

- Size:

  - Termination – Through all.

- Click ok.

- Create new 2d sketch for matching size holes and repeat above steps with appropriate types and sizes ect.

- Create new 2d sketch on back side of plate.

- Place 2 points.

- Dimension into correct position.

- Select hole tool and change parameters to:

- Type – simple hole.

- Seat – Counterbore.

- Termination through.

- Counterbore dimensions:

  - Head Ø = 10mm

  - Head depth = 5.1mm

  - Hole size = 5.5mm

Creating slot in base and chamfered corners

- Create 2d sketch on front face.

- Select rectangle tool and draw rectangular shape inside block, left click to form shape and then right click and press ok.

- Use dimensions tool to position and size.

- Finish sketch.

- Select extrude tool.

- Select rectangular shape as profile.

- Select cut away feature and set depth to 4mm.

- Select chamfer tool.

- Set distance to 5mm.

- Select corners to be chamfered.

- Click apply.

Creating stepped slot

- Create new 2d sketch on front side of block.

- Select rectangle tool.

- Draw required rectangle shape inside block.

- Dimension and position hole with dimension tool.

- Finish sketch.

- Extrude to full depth of block using cut feature.

- Select 2d sketch on back face.

- Draw new rectangle as per dimensions around existing cutout and dimension as appropriate.

- Finish sketch.

- Extrude hole back using cut feature.

- Select fillet tool.

- Set radius to 3mm and select edges to be filleted and click apply.

- Save file to correct file location under MV001.